BOROUGH OF CALIFON

ORDINANCE 2025-05

AN ORDINANCE AMENDING THE REVISED BOROUGH CODE OF THE BOROUGH OF CALIFON BY REPLACING SECTIONS 8.28.010, 8.28.040, AND 8.28.070, OF TITLE 8, CHAPTER 28, "SMOKE-SENSITIVE ALARM, CARBON MONOXIDE DEVICES AND FIRE EXTINGUISHERS", IN THEIR ENTIRETY

BE IT ORDAINED by the Borough Council of the Borough of Califon that the Borough Code adopted August 24, 2000 and heretofore amended be further amended as follows:

SECTION ONE:

Title 8 of the Borough Code, "Health and Safety", Chapter 8.28, "Smoke-Sensitive Alarm, Carbon Monoxide Devices and Fire Extinguishers", Section 8.28.010, "Smoke-sensitive devices required", is hereby amended by replacing it in its entirety with the following:

8.28.010 - Smoke alarms, carbon monoxide alarms, and portable fire extinguishers required.

A. Before any single family, two family or multifamily dwelling unit is sold, leased or otherwise made subject to a change of occupancy for residential purposes, the owner shall obtain a certificate of smoke alarm, carbon monoxide alarm, and portable fire extinguisher compliance (CSACMAPFEC certificate).

B. No dwelling unit shall change occupancy until one or more smoke alarms and one or more carbon monoxide alarms, or another approved fire alarm system, are installed for each dwelling unit in accordance with all the requirements of the New Jersey Uniform Construction Code and the Uniform Fire Code.

C. No dwelling unit shall change occupancy until at least one residential portable fire extinguisher is installed in accordance with the following:

1. An extinguisher shall be within 10 feet of the kitchen and located in the path of egress;

2. The extinguisher shall be readily accessible and not obstructed from view;

3. The extinguisher shall be mounted using the manufacturer's hanging bracket so the operating instructions are clearly visible; 25cg-03 2 June 17, 2025

4. The top of the extinguisher shall not be more than five feet above the floor.

5. The extinguisher shall be an approved listed and labeled type with a minimum rating of 2A-10B:C and no more than 10 pounds;

6. The owner's manual or written operation instructions shall be provided during the inspection and left for the new occupant; and

7. The extinguisher shall be serviced and tagged by a certified Division of Fire Safety contractor within the past 12 months or the seller must have a receipt for the purchase of the extinguisher within the past 12 months.

D. The fees for CSACMAPFEC certificates shall be as follows:

1. Requests for a CSACMAPFEC received more than 10 business days prior to a change of occupancy: $40;

2. Requests for a CSACMAPFEC received four to 10 business days prior to a change of occupancy: $70;

3. Requests for a CSACMAPFEC received fewer than four business days prior to a change of occupancy: $125. 4. The fee for each additional inspection after a failed inspection shall be the same as the fee for the initial inspection.

E. Failure to obtain a CSACMAPFEC Certificate shall result in a penalty of $250.

SECTION TWO:

Title 8 of the Borough Code, "Health and Safety", Chapter 8.28, "Smoke-Sensitive Alarm, Carbon Monoxide Devices and Fire Extinguishers", Section 8.28.040, "Change of occupancy", is hereby amended by replacing it in its entirety with the following:

8.28.040 – Reserved.

SECTION THREE:

Title 8 of the Borough Code, "Health and Safety", Chapter 8.28, "Smoke-Sensitive Alarm, Carbon Monoxide Devices and Fire Extinguishers", Section 8.28.070, "Inspections; fees ", is 25cg-03 3 June 17, 2025 hereby amended by replacing it in its entirety with the following:

8.28.070 – Reserved.

SECTION FOUR:

Effective Date. This Ordinance shall take effect immediately upon publication of Notice of Final Passage in the manner provided by law.

ATTEST:

___________________________			____________________________

Caitlin Haughey, RMC			Charles Daniel

Borough Clerk/Administrator			Mayor

Prepared by Mark S. Anderson, Borough Attorney

FIRST READING: June 17, 2025

PUBLICATION: June 25, 2025

PUBLIC HEARING/FINAL ADOPTION: July 15, 2025

PUBLICATION BY TITLE: July 23, 2025

CERTIFICATION

I certify that the foregoing is a true and accurate copy of a Resolution adopted by the Borough of Califon Council a regular meeting held on June 17, 2025.

(SEAL)

____________________________

Caitlin Haughey, RMC

Municipal Clerk/Administrator